/*!

\page SimTracker_SiPixelDigitizer Package SimTracker/SiPixelDigitizer

<center>

<small>

<!-- @PROJ_VERS@ will be substituted at build time, no need to touch -->

<a href=http://cmsdoc.cern.ch/swdev/viewcvs/viewcvs.cgi/CMSSW/SimTracker/SiPixelDigitizer/?cvsroot=CMSSW&only_with_tag=@PROJ_VERS@>CVS</a> -

<a href=http://cmsdoc.cern.ch/swdev/viewcvs/viewcvs.cgi/CMSSW/SimTracker/SiPixelDigitizer/.admin/developers?rev=HEAD&cvsroot=CMSSW&content-type=text/vnd.viewcvs-markup>Administrative privileges</a>

</small>

</center>

\section desc Description

<!-- Short description of what this package is supposed to provide -->

This package converts the simulated pixel hits to pixel digis.

The pixel digi information consists of the pixel row index, column index

and the adc value of the charge collectd by this pixel.

\subsection interface Public interface

<!-- List the classes that are provided for use in other packages (if any) -->

SiPixelDigitizer

The steering class. Gets the pixel simHits, loops over all pixel

detector modules and stores the digis.

SiPixelDigitizerAlgorithm

The main algorithm to digitze simhits in one detector module.

PixelIndices

A helper class which transforms pixel module/plaquet pixel indices

to ROC and DCOL indices. Can be used outside the digitizer.

\subsection modules Modules

<!-- Describe modules implemented in this package and their parameter set -->

SiPixelDigitizer

The following parameters are used by this module

tracker parameters defined in the configuration file :

double ElectronPerAdc=135.0           - calibration electrons/adc

double NoiseInElectrons =500.0        - pixel noise in electrons

double ThresholdInNoiseUnits = 5.     - pixel threshold in noise

double TanLorentzAnglePerTesla =0.106 - Lorentz angle per unit B field

int32 AddPixelInefficiency = 0        - pixel inefficiency

-1 - none

0 - static loss only, v.low lumi

2 - low lumi

10 - high lumi

bool AddNoise = true                  - add noise to the pixel signal

bool AddNoisyPixels = true            - add pixels generated by noise

bool MissCalibrate = false            - misscalibration the pixel gain

double GainSmearing = 0.0             - misscalibration spread in gain

double OffsetSmearing = 0.0           - misscalibration spread in offset

untracked parameters with default values provided :

int AdcFullScale  = 255               - adc full scale (8 bits)

double TofCut     = 12.5              - time of flight cut in nanoseconds

bool FluctuateCharge = true           - fluctuate generated charge along the track

double DeltaProductionCut = 0.030     - the delta production cut in MeV,

(has to be equal to the value used in OSCAR)

\subsection tests Unit tests and examples

<!-- Describe cppunit tests and example configuration files -->

The configuration file is in data/PixelDigi.cfi.

\section status Status and planned development

<!-- e.g. completed, stable, missing features -->

Stable.

Correspods rougly to the functinality which was available in ORCA.

Missing fatures: DB access for some of the parameters.

<hr>

Last updated:

@DATE@  Author: computer-generated.

*/